College of Engineering

Electrical and Computer Engineering Department

CCEE 454 – Computer Networks

Fall 2025

Project Report:

Campus Departments System Network Design

Project Report Submitted by:

Yara Ismail– 20230321

Nour Zoghat – 20210428

Anthony Abou Merhi– 20220568

Project Report Submitted to:

Ms. Amal El Arid

25 November, 2025

Section 1: Network Topology

Section 2: Full Addressing Table

| Device Name | Interface | IP Address | Subnet Mask | Default Gateway | MAC Address |
| --- | --- | --- | --- | --- | --- |
| Router | G0/0.10 | 192.168.1.1 | 255.255.255.224 | N/A |  |
|  | G0/0.20 | 192.168.1.33 | 255.255.255.240 | N/A |  |
|  | G0/0.30 | 192.168.1.49 | 255.255.255.240 | N/A |  |
| EngPC1 | Fa0 | 192.168.1.2 | 255.255.255.224 | 192.168.1.1 | 0002.4A73.0A85 |
| AdmPC1 | Fa0 | 192.168.1.34 | 255.255.255.240 | 192.168.1.33 | 000B.BE8A.17D6 |
| AdmLaptop | Fa0 | 192.168.1.43 | 255.255.255.240 | 192.168.1.33 | 0002.168A.696B |
| AdmServer | Fa0 | 192.168.1.40 | 255.255.255.240 | 192.168.1.33 | 000C.85E9.2938 |
| LibPC1 | Fa0 | 192.168.1.50 | 255.255.255.240 | 192.168.1.49 | 00E0.B0DC.99CC |

Section 3: Objectives

The University Campus’s network is designed with scalability, efficiency, and security in mind, ensuring seamless communication between all 3 departments. Each department has its own local area network (LAN) connected through dedicated switches each, which are then integrated into the overall network via a switch that is connected to a single router. The following is a breakdown of the network infrastructure:

Engineering LAN:

Department: Engineering

Devices:

- 24 PCs as workstations

- 1 Switch for communication within the network and the router.

Administration LAN :

Department: Administration

Devices:

- 8 PCs as workstations

- 1 Laptop

- 1 Server

- 1 Switch for communication within the network and the router.

Library LAN:

Department: Library

Devices:

- 4 PCs as workstations.

- 2 laptops

- 1 Switch for communication within the network and the router.

This hierarchical network design ensures efficient traffic management, simplifies troubleshooting, and provides room for future expansion as the departments grows.

Section 4: Network Design

The campus’s network design incorporates various networking devices to ensure seamless connectivity and communication across the three departments. Below is a detailed list of the devices used and their specifications, as per the provided topology image:

First Department Devices (Engineering):

The engineering department network includes 24 PCs as workstations, all configured to receive IP addresses via static. All devices are connected to a switch which serves as an intermediary between the devices and the switch that is connected to the router. The router manages static addressing and functions as the network gateway, with an IP address of 192.168.1.1

Second Department Devices (Administration):

The Administration department network includes 8 PCs as workstations, 1 Laptop and 1 Server, all configured to receive IP addresses via static. All devices are connected to a switch which serves as an intermediary between the devices and the switch connected to the router. The router manages static addressing and functions as the network gateway, with an IP address of 192.168.1.33

Third Department Devices (Library):

The Library department network includes 4 PCs as workstations and 2 Laptops, all configured to receive IP addresses via static. All devices are connected to a switch, which links them to the  router. The router manages static addressing and functions as the network gateway, with an IP address of 192.168.1.49

Section 5: Implementation Description

Router configuration

Central switch configuration

Department switches

Engineering switch configuration

Administration switch configuration

Library switch configuration

Section 6: Connection Verification

After setting up the network, we wanted to make sure everything was working smoothly, so we ran a few key tests:

Ping Test:
First, we ran ping tests between the PCs on the same department  to check if they could communicate with each other. Then, we pinged the PCs to the web sever to ensure the web server was accessible to all pcs . To check if the network was working between departments, we also pinged devices across the switches . This helped us confirm that the devices on different departments were able to connect and communicate without any issues.

Engineering pc ping test and web server accesses

Administration pc ping test and web server accesses

Library pc ping test and web server access